TUTORIAL

Getting started with ThingsIO.AI

- Register the account in http://thingsio.ai/#/register.

- Create a project:

- Write the name of the project:

- Now, you will be on the project dashboard of your account. Click on the new project option:

- Click on the add a new device option:

- Enter the device name and click on the create and configure device:

- You will be on your device dashboard:

- Track your device’s special parameters here. This can be set in device configuration (default to null).

- You can see your real time and special parameter’s graph.

- You can see your all data points.

- You can see here your last 5 data points.

- You can see here all the list of created graphs.

- Go in sample device code options and click on the CC3200. You will get the sample code from there and paste into your CCS.

- Click on the “Send trial data” to send a trial data to the server:

- Now, click on the configuration device option.

- You can set the special parameters (this is tracked on the device dashboard) & transform it accordingly and add new parameters (Parameters are updated automatically as you send them from your device).

- Click on the update device option:

- You can also set your device geographical location.

- To create a graph for your device click on the “create graph option”.

- Click on the preview option. You will see your created graph and click save changes.

TEMPERATURE SENSOR WITH NODE MCU

PLATFORM: WINDOWS

IDE: ARDUINO

CHIP: ESP8266 MOD WiFi  MODULE

1.LIBRARIES INCLUDED

a)  #include <SoftwareSerial.h>

- Install arduino IDE

- Go to sketch>include library>library manager>search software serial>install

- Checkout the  link provided for the detailed video on how to setup Arduino tool chain and libary for ESP8266:  https://youtu.be/ftO-_nfBBZk

b) #include <ESP8266WiFi.h>

- (https://github.com/esp8266/Arduino  )

c)  #include <DNSServer.h>

- Go to sketch>include library>library manager>search DNSserver>install

d) #include <ESP8266WebServer.h>

- Go to sketch>include library>library manager>search >install

e) #include "WiFiManager.h"

- (https://github.com/tzapu/WiFiManager)

2. CODE MODIFICATIONS

a) define the host

const char* host = "api.thingsio.ai";

b) define the post url

const char* post_url = "/devices/deviceData";

3. TEMPERATURE SENSING AND CONVERSION

switch(qry[3])

{

1) case 0x20:

t1=(analogValue/1024.0) * 3300;      (analogValue = analogRead(outputpin),

(outputpin= A0);

Serial.print("temp: ");

Serial.println(t1);

break;

2) case 0x21:

do the following  to obtain the Celsius value,

t2=t1/10;

Serial.print("celc: ");

Serial.println(t2);

break;

3) case 0x22:

t3=((t2 * 9)/5 + 32);

celcius to Fahrenheit conversion

Serial.print("fhrn: ");

Serial.println(t3);

break;

*You can modify the code as per the requirements and the sensor used

provide the device id as well as the slave id

String PostValue = "{\"device_id\": 201840, \"slave_id\": 2";

Post the values

PostValue = PostValue + ",\"dts\":" +timestamp;

PostValue = PostValue +",\"data\":{\"celc\":" + t2 +",\"fahr\":" + t3 +"}"+"}";

- Celc  temperature in celcius

- Fahrtemperature in Fahrenheit